Circuit Analysis Lab #10

Experiment: Verification of Thevenn’s Theorem

Objective

To verify Thevenin’s theorem and calculate the load current using Thevenin equivalent voltage and resistance.

Theory

Thevenin’s theorem states that any linear bilateral network can be replaced with an equivalent voltage source (VTH) in series with an equivalent resistance (RTH).

Circuit Components

Regulated Power Supply

Digital Multimeter

Breadboard and wires

1kΩ resistor

330Ω resistors

Potentiometer

Procedure

Connect the circuit according to the given diagram.

Set the DC voltage using regulated power supply.

Measure load current IL using ammeter.

Remove load resistor and measure open circuit voltage VTH.

Short the supply source and measure equivalent resistance RTH.

Construct Thevenin equivalent circuit and verify the current.

Assumed Practical Values

| E(V) | VTH(V) | RTH(Ω) | IL(mA) | Equivalent IL(mA) |
| --- | --- | --- | --- | --- |
| 12V | 8.9V | 248Ω | 15mA | 14.8mA |

Calculations

Given:
VTH = 8.9V
RTH = 248Ω
RL = 330Ω

Load Current:
IL = VTH / (RTH + RL)
IL = 8.9 / (248 + 330)
IL = 8.9 / 578
IL = 0.015A = 15mA

Result

The practical load current and equivalent circuit current are nearly equal. Hence, Thevenin’s theorem is verified.

Post Lab Questions

1. What is Thevenin’s Theorem?

Thevenin’s theorem states that any linear electrical network can be replaced by a single voltage source in series with an equivalent resistance.

2. Applications of Thevenin’s Theorem

Simplifies complex circuits.

Used in load analysis.

Helps in power transfer calculations.

Useful in circuit design and troubleshooting.

3. Limitations of Thevenin’s Theorem

Applicable only to linear circuits.

Not suitable for non-linear elements like diodes and transistors.

Dependent sources require special handling.